/*

IoT Anti-Theft System with Web App Integration

This sketch enables two-way communication with a web application to create a

complete anti-theft system.

Features:

- Connects to Wi-Fi.

- Polls a web server to get the system's armed/disarmed status.

- When armed, monitors ADXL345 accelerometer and a vibration sensor.

- If motion is detected, it activates a buzzer and sends an alert with GPS

coordinates and motion data to a web server endpoint.

- Continuously reads GPS data in the background.

Libraries Needed:

- ESP8266WiFi

- ESP8266HTTPClient

- ArduinoJson (by Benoit Blanchon)

- TinyGPS++ (by Mikal Hart)

- Adafruit ADXL345 (by Adafruit)

- Adafruit Unified Sensor (by Adafruit)

- SoftwareSerial

*/

// --- LIBRARIES ---

#include <ESP8266WiFi.h>

#include <ESP8266HTTPClient.h>

#include <ArduinoJson.h>      // For handling JSON data with the web app

#include <SoftwareSerial.h>   // For GPS module

#include <TinyGPS++.h>         // For parsing GPS data

#include <Wire.h>              // For I2C communication (ADXL345)

#include <Adafruit_ADXL345_U.h>

#include <Adafruit_Sensor.h>

// --- CONFIGURATION ---

const char* ssid = "YOUR_WIFI_SSID";         // Replace with your Wi-Fi network name

const char* password = "YOUR_WIFI_PASSWORD"; // Replace with your Wi-Fi password

// !!! IMPORTANT: REPLACE WITH YOUR WEB APP URLS !!!

const char* getStatusUrl = "http://your-web-app.com/api/getstatus?deviceId=123";

const char* sendAlertUrl = "http://your-web-app.com/api/alert";

// --- PIN DEFINITIONS ---

#define VIBRATION_PIN   D5  // GPIO14 - Digital Out (DO) of the vibration sensor

#define BUZZER_PIN      D6  // GPIO12 - To control the buzzer or a relay

#define GPS_RX_PIN      D7  // GPIO13 - Connect to the TX pin of the GPS module

#define GPS_TX_PIN      D8  // GPIO15 - Connect to the RX pin of the GPS module

// --- GLOBAL OBJECTS & VARIABLES ---

TinyGPSPlus gps;

SoftwareSerial ss(GPS_RX_PIN, GPS_TX_PIN);

Adafruit_ADXL345_Unified accel = Adafruit_ADXL345_Unified(12345);

// System state machine

enum SystemState { DISARMED, ARMED, TRIGGERED };

SystemState currentState = DISARMED;

// Timing for non-blocking loops

unsigned long lastStatusCheck = 0;

const long statusCheckInterval = 5000; // Check web app every 5 seconds

// ===============================================================

// SETUP: Runs once

// ===============================================================

void setup() {

Serial.begin(115200);

Wire.begin();       // Init I2C for ADXL345 (SDA=D2, SCL=D1)

ss.begin(9600);     // Init serial for GPS

pinMode(VIBRATION_PIN, INPUT_PULLUP);

pinMode(BUZZER_PIN, OUTPUT);

digitalWrite(BUZZER_PIN, LOW); // Buzzer off

Serial.println("\n--- Anti-Theft System Initializing ---");

connectToWiFi();

if (!accel.begin()) {

Serial.println("Error: ADXL345 sensor not found!");

while (1);

}

accel.setRange(ADXL345_RANGE_4_G);

Serial.println("ADXL345 Initialized.");

Serial.println("Setup complete. Awaiting commands from server...");

}

// ===============================================================

// LOOP: Runs continuously

// ===============================================================

void loop() {

// Always listen for GPS data in the background

while (ss.available() > 0) {

gps.encode(ss.read());

}

// Periodically check the web app for the system's status

if (millis() - lastStatusCheck > statusCheckInterval) {

checkSystemStatus();

lastStatusCheck = millis();

}

// --- Main Logic based on State ---

if (currentState == ARMED) {

// Read the vibration sensor state (active LOW)

int vibrationState = digitalRead(VIBRATION_PIN);

if (vibrationState == LOW) {

Serial.println("!!! MOTION DETECTED !!!");

currentState = TRIGGERED;

digitalWrite(BUZZER_PIN, HIGH); // Turn buzzer ON

sendAlertToWebApp();

}

}

}

// ===============================================================

// --- Custom Functions ---

// ===============================================================

void connectToWiFi() {

Serial.print("Connecting to WiFi...");

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nWiFi connected! IP: " + WiFi.localIP().toString());

}

/**

* @brief Checks the web app to get the current desired state (ARMED/DISARMED).

*/

void checkSystemStatus() {

HTTPClient http;

WiFiClient client;

Serial.print("Checking system status from server... ");

http.begin(client, getStatusUrl);

int httpCode = http.GET();

if (httpCode > 0) {

if (httpCode == HTTP_CODE_OK) {

String payload = http.getString();

Serial.println("Payload: " + payload);

// Parse the JSON response

JsonDocument doc;

DeserializationError error = deserializeJson(doc, payload);

if (error) {

Serial.print("deserializeJson() failed: ");

Serial.println(error.c_str());

return;

}

const char* status = doc["status"]; // Expects JSON like: {"status": "ARMED"}

if (strcmp(status, "ARMED") == 0) {

if (currentState == DISARMED) {

Serial.println("System state changed to: ARMED");

currentState = ARMED;

}

} else { // Assumed DISARMED

if (currentState != DISARMED) {

Serial.println("System state changed to: DISARMED");

currentState = DISARMED;

digitalWrite(BUZZER_PIN, LOW); // Turn buzzer OFF

}

}

}

} else {

Serial.printf("GET failed, error: %s\n", http.errorToString(httpCode).c_str());

}

http.end();

}

/**

* @brief Sends an alert to the web app with sensor and location data.

*/

void sendAlertToWebApp() {

HTTPClient http;

WiFiClient client;

http.begin(client, sendAlertUrl);

http.addHeader("Content-Type", "application/json");

// Get accelerometer data

sensors_event_t event;

accel.getEvent(&event);

// Create JSON document

JsonDocument doc;

doc["deviceId"] = "123"; // Should match the deviceId in the URL

doc["type"] = "VibrationAlert";

// Add GPS data

if (gps.location.isValid()) {

doc["latitude"] = gps.location.lat();

doc["longitude"] = gps.location.lng();

} else {

doc["latitude"] = 0;

doc["longitude"] = 0;

}

// Add accelerometer data

doc["accelX"] = event.acceleration.x;

doc["accelY"] = event.acceleration.y;

doc["accelZ"] = event.acceleration.z;

String requestBody;

serializeJson(doc, requestBody);

Serial.println("Sending alert to server: " + requestBody);

int httpCode = http.POST(requestBody);

if (httpCode > 0) {

String response = http.getString();

Serial.println("Server response: " + response);

} else {

Serial.printf("POST failed, error: %s\n", http.errorToString(httpCode).c_str());

}

http.end();

}