Modified Code [almost complete]

// --- LIBRARIES ---

#include <ESP8266WiFi.h>

#include <ESP8266HTTPClient.h>

#include <ArduinoJson.h>

#include <SoftwareSerial.h>

#include <TinyGPS++.h>

#include <Wire.h>

#include <Adafruit_ADXL345_U.h>

#include <Adafruit_Sensor.h>

// --- CONFIGURATION ---

const char* ssid = "YOUR_WIFI_SSID";

const char* password = "YOUR_WIFI_PASSWORD";

// Web App URLs - Update these to match your server

const char* getStatusUrl = "http://your-server-ip:3000/api/getstatus?deviceId=123";

const char* sendAlertUrl = "http://your-server-ip:3000/api/alert";

// --- PIN DEFINITIONS ---

#define VIBRATION_PIN   D5  // SW420 vibration sensor

#define BUZZER_PIN      D6  // Buzzer

#define GPS_RX_PIN      D7  // GPS TX -> ESP RX

#define GPS_TX_PIN      D8  // GPS RX -> ESP TX

#define RED_LED_PIN     D2  // Red LED for armed state

#define GREEN_LED_PIN   D3  // Green LED for disarmed state

// --- GLOBAL OBJECTS & VARIABLES ---

TinyGPSPlus gps;

SoftwareSerial ss(GPS_RX_PIN, GPS_TX_PIN);

Adafruit_ADXL345_Unified accel = Adafruit_ADXL345_Unified(12345);

// System state machine

enum SystemState { DISARMED, ARMED, TRIGGERED };

SystemState currentState = DISARMED;

// Timing for non-blocking operations

unsigned long lastStatusCheck = 0;

const long statusCheckInterval = 5000; // Check status every 5 seconds

// ===============================================================

// SETUP: Runs once

// ===============================================================

void setup() {

Serial.begin(115200);

Wire.begin();       // Init I2C for ADXL345

ss.begin(9600);     // Init software serial for GPS

// Initialize pins

pinMode(VIBRATION_PIN, INPUT_PULLUP);

pinMode(BUZZER_PIN, OUTPUT);

pinMode(RED_LED_PIN, OUTPUT);

pinMode(GREEN_LED_PIN, OUTPUT);

// Initial state - disarmed

digitalWrite(BUZZER_PIN, LOW);

digitalWrite(RED_LED_PIN, LOW);

digitalWrite(GREEN_LED_PIN, HIGH);

Serial.println("\n--- PedalGuard Initializing ---");

connectToWiFi();

// Initialize accelerometer

if (!accel.begin()) {

Serial.println("Error: ADXL345 sensor not found!");

while (1);

}

accel.setRange(ADXL345_RANGE_4_G);

Serial.println("ADXL345 Initialized.");

Serial.println("Setup complete. Awaiting commands from server...");

}

// ===============================================================

// LOOP: Runs continuously

// ===============================================================

void loop() {

// Always listen for GPS data in the background

while (ss.available() > 0) {

gps.encode(ss.read());

}

// Periodically check the web app for system status

if (millis() - lastStatusCheck > statusCheckInterval) {

checkSystemStatus();

lastStatusCheck = millis();

}

// --- Main Logic based on State ---

switch (currentState) {

case ARMED:

monitorSensors();

break;

case TRIGGERED:

// Keep buzzing until disarmed

break;

case DISARMED:

// Just wait for status changes

break;

}

}

// ===============================================================

// --- CUSTOM FUNCTIONS ---

// ===============================================================

void connectToWiFi() {

Serial.print("Connecting to WiFi...");

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nWiFi connected! IP: " + WiFi.localIP().toString());

}

void checkSystemStatus() {

if (WiFi.status() != WL_CONNECTED) {

Serial.println("WiFi disconnected. Reconnecting...");

connectToWiFi();

return;

}

HTTPClient http;

WiFiClient client;

Serial.print("Checking system status from server... ");

http.begin(client, getStatusUrl);

int httpCode = http.GET();

if (httpCode > 0) {

if (httpCode == HTTP_CODE_OK) {

String payload = http.getString();

Serial.println("Payload: " + payload);

// Parse the JSON response

JsonDocument doc;

DeserializationError error = deserializeJson(doc, payload);

if (error) {

Serial.print("deserializeJson() failed: ");

Serial.println(error.c_str());

http.end();

return;

}

const char* status = doc["status"]; // Expects JSON like: {"status": "ARMED"}

if (strcmp(status, "ARMED") == 0) {

if (currentState == DISARMED || currentState == TRIGGERED) {

Serial.println("System state changed to: ARMED");

currentState = ARMED;

digitalWrite(RED_LED_PIN, HIGH);

digitalWrite(GREEN_LED_PIN, LOW);

digitalWrite(BUZZER_PIN, LOW); // Ensure buzzer is off when arming

}

} else { // Assumed DISARMED

if (currentState != DISARMED) {

Serial.println("System state changed to: DISARMED");

currentState = DISARMED;

digitalWrite(RED_LED_PIN, LOW);

digitalWrite(GREEN_LED_PIN, HIGH);

digitalWrite(BUZZER_PIN, LOW); // Turn buzzer OFF

}

}

}

} else {

Serial.printf("GET failed, error: %s\n", http.errorToString(httpCode).c_str());

}

http.end();

}

void monitorSensors() {

// Read the vibration sensor state (active LOW)

int vibrationState = digitalRead(VIBRATION_PIN);

if (vibrationState == LOW) {

Serial.println("!!! VIBRATION DETECTED !!!");

triggerAlarm();

}

// Check accelerometer for sudden movements

sensors_event_t event;

accel.getEvent(&event);

// Calculate total acceleration (excluding gravity)

float accelTotal = sqrt(pow(event.acceleration.x, 2) +

pow(event.acceleration.y, 2) +

pow(event.acceleration.z, 2));

// If acceleration exceeds threshold (adjust as needed)

if (accelTotal > 2.5) { // 2.5G threshold

Serial.println("!!! SUDDEN MOVEMENT DETECTED !!!");

triggerAlarm();

}

}

void triggerAlarm() {

if (currentState != ARMED) return; // Only trigger if armed

currentState = TRIGGERED;

digitalWrite(BUZZER_PIN, HIGH); // Turn buzzer ON

sendAlertToWebApp();

// Blink red LED rapidly

for (int i = 0; i < 10; i++) {

digitalWrite(RED_LED_PIN, !digitalRead(RED_LED_PIN));

delay(200);

}

}

void sendAlertToWebApp() {

if (WiFi.status() != WL_CONNECTED) {

Serial.println("Cannot send alert - WiFi disconnected");

return;

}

HTTPClient http;

WiFiClient client;

http.begin(client, sendAlertUrl);

http.addHeader("Content-Type", "application/json");

// Get accelerometer data

sensors_event_t event;

accel.getEvent(&event);

// Create JSON document

JsonDocument doc;

doc["deviceId"] = "123"; // Should match your device ID

doc["type"] = "TheftAlert";

doc["timestamp"] = millis();

// Add GPS data if available

if (gps.location.isValid()) {

doc["latitude"] = gps.location.lat();

doc["longitude"] = gps.location.lng();

doc["speed"] = gps.speed.kmph();

doc["altitude"] = gps.altitude.meters();

} else {

doc["latitude"] = 0;

doc["longitude"] = 0;

doc["speed"] = 0;

doc["altitude"] = 0;

}

// Add sensor data

doc["vibration"] = true;

doc["accelX"] = event.acceleration.x;

doc["accelY"] = event.acceleration.y;

doc["accelZ"] = event.acceleration.z;

String requestBody;

serializeJson(doc, requestBody);

Serial.println("Sending alert to server: " + requestBody);

int httpCode = http.POST(requestBody);

if (httpCode > 0) {

String response = http.getString();

Serial.println("Server response: " + response);

} else {

Serial.printf("POST failed, error: %s\n", http.errorToString(httpCode).c_str());

}

http.end();

}

// Add this to your existing Arduino code

void handleLockCommand(bool lock) {

if (lock) {

currentState = ARMED;

digitalWrite(RED_LED_PIN, HIGH);

digitalWrite(GREEN_LED_PIN, LOW);

Serial.println("System ARMED via remote command");

} else {

currentState = DISARMED;

digitalWrite(RED_LED_PIN, LOW);

digitalWrite(GREEN_LED_PIN, HIGH);

digitalWrite(BUZZER_PIN, LOW);

Serial.println("System DISARMED via remote command");

}

}

// Modify checkSystemStatus() to handle lock commands

void checkSystemStatus() {

HTTPClient http;

WiFiClient client;

http.begin(client, getStatusUrl);

int httpCode = http.GET();

if (httpCode == HTTP_CODE_OK) {

String payload = http.getString();

JsonDocument doc;

deserializeJson(doc, payload);

const char* command = doc["command"]; // "lock" or "unlock"

if (strcmp(command, "lock") == 0) {

handleLockCommand(true);

} else if (strcmp(command, "unlock") == 0) {

handleLockCommand(false);

}

}

http.end();

}